| Periodized Table of Stratigraphic Units | Building/Site: Church of São João de Calvos (Guimarães) | Recorder: Research team | Date: 12/2024 |
| --- | --- | --- | --- |

| Phase | A (Activities) | SU | Name/brief description | Prior to | Contemporary to | Later than |
| --- | --- | --- | --- | --- | --- | --- |
| Phase Ia 12th–13th centuries | A100 Main chapel and beginning of the north-western façade | 001 | Wall of the main chapel, built in granite ashlar masonry arranged in horizontal courses, finished with a cavetto cornice supported by six cavetto corbels. They are decorated with spheres and geometric motifs on its north-western and south-eastern façades. The north-eastern elevation, contains a slit window. | 002, 003, 004, 005, 006, 008, 009, 012, 045, 046, 047?, 048, 049, 050, 051, 052, 053, 054, 055, 056, 057, 058, 059, 060, 061, 079, 104, 105, 108, 109, | 043 |  |
|  |  | 007 | Antefixed stone cross on the north-eastern wall of the main chapel. | 006 |  | 001 |
|  |  | 043 | Remains from the beginning of the north-western façade of the nave, built in granite ashlar masonry that continues the courses of the apse. | 006, 010, 042, 060, 061, 099, 117 | 001 |  |
|  |  | 113 | Original wall of the presbytery, above the current pointed arch, built in granite ashlar masonry laid in horizontal courses. | 012, 050, 063, 096, 098, 099 | 001, 043 |  |
|  | A101 Floor of the Main Chapel | 104 | Stone flooring in the chancel. Elements with different shapes and sizes. | 062, 103, 105 |  |  |
|  |  | 105 | Raised stone platform on the chancel floor. | 107 |  | 104 |
|  | A102 Altar base | 107 | Altar stand/ reused Roman ara. |  |  | 105 |
| Fase Ib 13th-14th centuries | A103 Nave of the church | 012 | Wall of the nave built in granite ashlar masonry arranged in undulating courses, finished on the north-western and south-eastern façades with a rectangular-profile cornice and cavetto corbels, undecorated and larger than those used in the apse. It contains three lintelled doorways on the north-western, south-eastern, and south-western façades and, above the south-western and north-western doors, a narrow slit window. The plan of the nave does not share the same orientation as the apse; it is rotated toward south. | 006, 010, 011, 015, 016, 019, 020, 021, 022, 024, 025, 026, 027, 028, 029, 030, 031, 034, 035, 036, 037, 038, 039, 040, 041, 060, 061, 063, 064, 065, 066, 067, 068, 069, 070, 071, 072, 074, 075, 076, 077, 078, 079, 085, 086, 087, 088, 089, 090, 091, 092, 093, 094, 095, 102, 119, 120, 121 |  | 001, 013, 042, 112, 113 |
|  |  | 013 | Foundation supporting the south-eastern façade of the nave (left side of the door). | 003, 012, 014 | 112 |  |
|  |  | 042 | Break interface of the primitive nave (SU043), contemporary with the apse SU001. | 012, 010 |  | 043 |
|  |  | 112 | Foundation supporting the south-eastern façade of the nave (right side of the door). | 003, 012, 014, 061 | 013 |  |
|  | A104 Renovation of the chancel arch | 060 | Pointed arch with voussoirs and the impost from which it rises, built in granite ashlar masonry. |  |  | 001, 012, 043, 061, 099 |
|  |  | 061 | Ashlar masonry pillars on which the presbytery arch rests. | 060 |  | 001, 012, 043, 099, 102 |
|  |  | 098 | Closing the interior window over the presbytery arch. | 113 |  |  |
|  |  | 099 | Interface - tear in the wall to place the SU060 arch on the south-western elevation of the nave. | 060 |  | 113 |
|  |  | 102 | Interface - cut -out in the nave floor, near the chancel, NE angle. | 061 |  | 101 |
|  | A105 Floor of the nave | 101 | Stone floor in the nave, different shape and dimensions, some elements reused. | 061, 062, 102, 103, 106, 119 |  | 012, 043, 112 |
|  | A106 Porch attached to the south-eastern façade of the nave | 015 | Console n.º 2, from west to east, on the nave south-eastern façade. | 017 |  | 012 |
|  |  | 016 | Console n.º 1, from west to east, on the nave south-eastern façade. | 018 |  | 012 |
|  |  | 017 | Interface - cut-out in console n.º 2 to fit the side porch support beam. |  |  | 015 |
|  |  | 018 | Interface - cut-out in console n.º 1 to fit the side porch support beam. |  |  | 016 |
|  |  | 019 | Interface - In-line holes under the cornice on the nave south-eastern façade. |  |  | 012 |
|  | A107 The Stations of the Cross carved | 079 | Crosses engraved (via sacra) on the ashlars inside the church (nave and main chapel). |  |  | 001, 012 |
|  | A108 Undetermined hole | 021 | Interface - hole in the south-eastern wall of the nave, in the fifth row, next to the chancel. | 022 |  | 012 |
|  | A109 Remains of the fixing of the first south-eastern and south-western doors of the nave | 075 | Interface - hole torn to insert hinges (SU076) from a previous door on the south-eastern elevation of the nave. Right side, lower part of the jamb. |  |  | 012 |
|  |  | 076 | Hinges of a former door on the south-eastern elevation of the nave. Right side, upper part of the jamb. |  |  | 012 |
|  |  | 083 | Interface - hole in the nave's south-western elevation, to the right of the access opening, in the fourth course of the jamb, possibly from an earlier door. |  |  | 012 |
|  |  | 091 | Interface - circular hole in the nave's south-western elevation, related to the iron metal element that reinforced the door's closure. Right side, possibly from an earlier door. |  |  | 012 |
| Phase II 15th–16th centuries | A200 Single-arch bell gable | 030 | Interface - cut-out in the cornice that lines the wall of the nave façade. | 031 |  | 012 |
|  |  | 031 | Supporting embankment related to the possible access staircase to the bell-gable. | 033 |  | 012, 030 |
|  |  | 032 | Single-arch bell gable with a triangular gable top, built in granite ashlar masonry. The arch rises from a chamfered impost. |  |  | 010, 012, 031 |
|  |  | 033 | Bell and its wooden support. |  |  | 032 |
|  | A201 Access to the belfry via the south-western façade | 029 | Interface - holes in the façade of the nave related to some structure that covered the entrance. | 034 |  | 012 |
|  | A202 Mark left by the bell rope rubbing against it | 027 | Interface - Abrasion mark on the nave façade produced by the rope used to ring the bell | 028 |  | 012 |
|  | A203 Renovation of the south-eastern door of the nave | 071 | Interface - circular hole cut into the south-eastern and south-western wall of the nave, to the right of the access opening, in the 4th row. |  |  | 012 |
|  |  | 072 | Interface (cut) in the south-eastern and south-western wall of the nave, to the left of the access opening, in the second and third courses. | 073 |  | 012 |
|  |  | 073 | Filling the SU072 interface. |  |  | 072 |
|  |  | 074 | Interface - cut in the south-eastern and south-western wall of the nave, to the right of the access opening, in the first courses. | 024 |  | 012 |
|  |  | 077 | Interface - cut-out on the south-eastern elevation of the nave. Right side, middle part of the jamb. | 024 |  | 012 |
|  |  | 081 | Interface - cut-out on the south-eastern elevation, west of the door. |  |  | 012 |
|  | A204 Renovation of the west door of the nave | 084 | Interface no. 1 – cut in the south-western elevation, on the right side of the door, from top to bottom, for the fixing of the wooden door. |  |  | 012 |
|  |  | 085 | Interface no. 2 – cut in the south-western elevation, on the right side of the door, from top to bottom, for the fixing of the wooden door. |  |  | 012 |
|  |  | 086 | Interface no. 3 – cut in the south-western elevation, on the right side of the door, from top to bottom, for the fixing of the wooden door. |  |  | 012 |
|  |  | 087 | Interface no. 1 – cut in the south-western elevation, on the left side of the door, from top to bottom, for the fixing of the wooden door. |  |  | 012 |
|  |  | 088 | Interface no. 2 – cut in the south-western elevation, on the left side of the door, from top to bottom, for the fixing of the wooden door. |  |  | 012 |
|  |  | 089 | Interface no. 3 – cut in the south-western elevation, on the left side of the door, from top to bottom, for the fixing of the wooden door. |  |  | 012 |
|  | A205 Renovation of the north-western door of the nave | 092 | Interface - repair opening/door in north-western elevation. Possible removal of a wooden lintel? | 093 |  | 012 |
|  | A206 Construction of a high choir at the western end of the nave | 078 | Interface - holes cut into the north-western elevation to support a high choir. |  |  | 012 |
|  |  | 120 | Negative related to the existence of a choir on the south-western elevation of the nave (internal side of the façade). |  | 121 | 012 |
|  |  | 121 | Negative related to the existence of a choir on the north-western elevation of the nave (internal side of the façade). |  | 120 | 012 |
|  | A207 Renovation of the chancel roof | 114 | Refurbishment of the east elevation of the chancel (internal side of the façade), built in granite ashlar masonry. |  |  | 001 |
|  | A208 Almsbox | 100 | Wooden alms box placed on the west elevation of the presbytery. |  |  | 061 |
| Phase IIIa Before 1952 | A300 Installation of a lighting system in the Main Chapel | 049 | Interface - circular holes torn in the exterior south-eastern wall of the main chapel (cable fixing). |  |  | 001 |
|  |  | 053 | Interface - perforation in the side walls of the main chapel (electricity cables). |  |  | 001 |
|  |  | 054 | Electrical outlet on the south-eastern elevation of the chancel. |  |  | 001 |
|  |  | 055 | Light switch, south-eastern elevation of the nave. |  |  | 001 |
|  |  | 066 | Light switch n.º 1 (W to E) in the south-eastern elevation of the nave. |  |  | 012 |
|  |  | 067 | Light switch n.º 2 (W to E) in the south-eastern elevation of the nave. |  |  | 012 |
|  |  | 108 | Lighting system of the chancel (2 lamps + chandelier), in the south-eastern elevation of the nave. | 050 |  | 001 |
|  |  | 116 | Light switch on the inner wall of the chancel, under the arch impost, on the west elevation of the presbytery. |  |  | 061 |
|  |  | 117 | Cut for fitting the SU118 power socket, on the west elevation of the presbytery. | 118 |  | 061 |
|  |  | 118 | Electricity socket to the north-western of the west elevation of the presbytery. |  |  | 117 |
|  | A301 Installation of a lighting system in the nave | 080 | Electrical outlet on the south-eastern elevation of the nave, west of the door. |  |  | 012 |
|  |  | 095 | Nave´s lighting system (bracket and 2 lamps) on the north-western and south-eastern elevation of the nave. |  |  | 012 |
|  |  | 119 | Tear in the paving stones of the nave, in the SW corner. | 103 |  | 101 |
|  | A302 Limewash finish on the walls | 009 | Traces of a white coating, possibly limewash or paint, identified on the cornice of the south-eastern elevation of the apse. |  |  | 001 |
|  |  | 059 | Traces of a whitish wall coating preserved at the upper section of the south-eastern elevation of the chancel/apse. |  |  | 001, 124 |
|  | A303 Cross and its wooden base | 056 | Wooden stand and cross on the south-eastern elevation of the chancel. |  |  | 001 |
| Phase IIIb First half of the 20th century | A304 Cracks caused by neglect | 004 | Damage/crack in the chancel's 3rd corbel (west to east). |  |  | 001 |
|  |  | 005 | Damage/crack in metope no. 1 (west to east) between chancels. |  |  | 001 |
|  |  | 028 | Damage/vertical crack in the lintel of the nave south-western façade door. |  |  | 012, 027 |
|  |  | 044 | Sub-horizontal damage/crack in the door lintel of the nave's south-eastern wall. |  |  | 012 |
|  |  | 046 | Damage/vertical crack in one of the ashlars of the north-western wall of the main chapel/apse (3rd row). |  |  | 001 |
|  |  | 111 | Lesion/crack in an ashlar block on the south-eastern wall of the nave (fourth course, north-eastern side). |  |  | 012 |
| Phase IIIc Second half of the 20th century | A305 Unspecified breaks and cuts and their repairs Second half of the 20th century | 022 | Stone that fills the hole SU021. |  |  | 021 |
|  |  | 034 | Stone filling one of the holes identified with SU029 on the façade of the nave |  |  | 029 |
|  |  | 038 | Interface - hole in the north-western wall of the nave, east of the door | 039 |  | 012 |
|  |  | 039 | Filling of SU038 with a stone and mortar. |  |  | 012, 038 |
|  |  | 040 | Interface - break over the door lintel of the north-western wall of the nave. | 041 |  | 012 |
|  |  | 041 | Break interface located above the lintel of the door in the north-western wall of the nave. |  |  | 012, 040 |
|  |  | 051 | Interface - hole in an ashlar of the south-eastern elevation (inside the main chapel). | 052 |  | 001 |
|  |  | 052 | Stone that closes the hole SU051. |  |  | 001, 051 |
|  |  | 057 | Interface - cut-out in the south-eastern elevation of the chancel, under the cross. | 058 |  | 001 |
|  |  | 058 | Filling of the cut-out SU057. |  |  | 001, 057 |
|  |  | 090 | Filling the SU089 interface. |  |  | 012, 089 |
|  |  | 098 | Closing the interior window over the presbytery arch. |  |  | 113 |
|  | A306 Opening windows in the nave and the installation of glass | 047 | Stone infill in the lower section of the arrow slit on the façade of the chancel (north-eastern wall). | 048 |  | 001 |
|  |  | 048 | Glass closing the arrow slit on the façade of the chancel (north-eastern wall). |  |  | 001, 047 |
|  |  | 065 | Interface for opening a window in the south-eastern elevation of the nave. | 070 |  | 012 |
|  |  | 070 | Glass that closes the window UE065. |  |  | 065 |
|  |  | 082 | Glass that closes the arrow slit in the south-western façade of the nave. |  |  | 012 |
|  |  | 094 | Glass that closes the arrow slit in the north-western façade of the nave. |  |  | 012 |
|  |  | 115 | Refurbishment of the original main chapel window (= SU047). | 048 | 047 | 001 |
|  | A307 Repair of the nave doors and installation of wooden locks | 024 | Rectangular wooden door in the opening of the south-eastern wall of the nave. |  |  | 012, 014, 074, 077 |
|  |  | 026 | Rectangular wooden door in the opening of the west wall on the nave. |  |  | 012, 084, 085, 086, 087, 088, 089 |
|  |  | 036 | Rectangular wooden door in the opening of the north-western wall of the nave. |  |  | 092 |
|  |  | 064 | Interface in the area of the door and lintel on the north-western elevation of the nave. |  |  | 012 |
|  |  | 093 | Interface - repair opening/door in north-western elevation of nave. | 036, 093 |  | 012 |
|  | A308 Repair of the church floor and the thresholds of the doorways | 002 | Cement coating between the lower edge of the outer wall and the pavement (half cane). |  |  | 001 |
|  |  | 003 | Stone pavement around the church. |  |  | 001, 012, 014, 025, 035, 042, 043 |
|  |  | 014 | Sill of the nave´s south-eastern wall opening. | 003, 024 |  | 012 |
|  |  | 025 | Sill of the nave´s south-western wall opening. | 003, 026 |  | 012 |
|  |  | 035 | Sill of the nave´s north-western wall opening. | 003, 036 |  | 012 |
|  |  | 106 | Cement that fills the joints in the main nave floor. | 103 |  | 101 |
|  |  | 062 | Sill between the main chapel and the nave, under the SU060 presbytery arch. | 103 |  | 061, 101, 104 |
|  | A309 Upgrade to the lighting system | 103 | Electricity cable in the ground of the church. |  |  | 012, 014, 025, 062, 101, 104, 106 |
|  | A310 Wooden roof | 010 | Oldest roof of the church nave. | 011 |  | 012, 031, 042, 043 |
|  |  | 050 | Main chapel roof timbering. | 006 |  | 001, 060, 113, 114 |
|  |  | 063 | Nave roof timbering. | 010, 011 |  | 012, 096, 097, 113 |
|  |  | 096 | Interface - cut-out for repairing the roof, over the presbytery arch. | 063, 097 |  | 113 |
|  |  | 097 | Filling of SU096 with a stone and mortar wall. |  |  | 096, 113 |
|  |  | 098 | Closing the interior window over the presbytery arch. |  |  | 113 |
|  | A311 Repairing joints with cement | 020 | Cement level that fills the joints in the external walls of the building. |  |  | 012 |
| Phase IIId Late 20th and early 21st centuries | A312 Roof renovation | 006 | Roof (tiles) of the main chapel/apse. |  |  | 001, 007, 009, 012, 050 |
|  |  | 008 | Cement level that waterproofs the roof in contact with the eaves. |  |  | 001, 006, 012, 043, 045 |
|  |  | 011 | Renovation of the nave roof in the area closest to the eaves. |  |  | 010, 012, 030, 031 |
|  |  | 045 | Metal sheet for waterproofing the chancel between the roof and cornice. |  |  | 001, 006, 008, 043 |
| Not linked to a phase | Not linked to an A | 021 | Interface - hole in the south-eastern wall of the nave, in the fifth row, next to the apse. | 022 |  | 012 |
|  |  | 023 | Interface - hole under the lintel of the opening in the south-eastern wall of the nave. |  |  | 012 |
|  |  | 037 | Interface - hole in the north-western wall of the nave, west of the door. |  |  | 012 |
|  |  | 038 | Interface - hole in the north-western wall of the nave, east of the door. | 039 |  | 012 |
|  |  | 040 | Interface - break in the door lintel of the north-western wall of the nave. | 041 |  | 012 |
|  |  | 051 | Interface - hole in an ashlar of the south-eastern elevation (inside the main chapel). | 052 |  | 001 |
|  |  | 068 | Recess in one of the ashlars on the north-western elevation of the nave, to the east of the access door. |  |  | 012 |
|  |  | 069 | Recess in one of the ashlars on the north-western elevation of the nave, to the west of the access door. | 078? |  | 012 |
|  |  | 109 | Interface - holes in the north-western elevation of the main chapel / apse. |  |  | 001 |
|  |  | 110 | Interface - tear in the south-eastern elevation of the main chapel / apse. |  |  | 001 |